TECHNICAL REPORT: RTIPVD

Real-Time Illegally Parked Vehicle Detection System

Indian Institute of Technology (IIT) Roorkee
Project Year: 2026
Document Classification: Technical Specification

1. Executive Summary & Overview

The Real-Time Illegally Parked Vehicle Detection (RTIPVD) system is an advanced, end-to-end computer vision and edge-computing pipeline designed to identify and log parking violations from dynamic camera feeds. Developed for deployment on mobile assets such as patrol vehicles, municipal drones, and public transit dashcams, the system addresses the critical urban challenge of monitoring illegal parking without relying on expensive, static roadside infrastructure.

By combining modern object detection (YOLOv8) and persistent multi-object tracking (ByteTrack) with advanced ego-motion compensation techniques, RTIPVD accurately distinguishes between stationary traffic and actual parking violations. The platform integrates localized Automatic License Plate Recognition (ALPR) and geospatial telemetry (GPS) to produce verifiable, geofenced, and database-ready violation logs in real time.

2. System Architecture & Core Components

The RTIPVD framework employs a distributed Edge-to-Server topology. This decouples resource-constrained telemetry and frame acquisition on the edge from intensive deep learning inference tasks performed on the local processing server.

| System Property | Technical Implementation / Details |
| --- | --- |
| Core Engine | Python 3.11 Environment |
| Computer Vision | YOLOv8 (Object Detection) + ByteTrack (Multi-Object Tracking) |
| Text Recognition | Regex-validated Automatic License Plate Recognition (ALPR / OCR) |
| Edge Hardware Node | Raspberry Pi Single Board Computer + ESP32 Microcontroller (with NEO-6M GPS) |
| Processing Server | Laptop or Cloud Infrastructure (Compatible with CPU and NVIDIA CUDA Acceleration) |
| Database Management | SQLite (Local relational database for transaction resilience) |
| User Interface / API | Flask / FastAPI Backend + Reactive HTML/JS/CSS Web Dashboard |

3. Core Technical Pipeline & Feature Set

Moving Camera Support: Compensates for feed instability from dashcams, drones, and patrol assets.

Ego-Motion Compensation: Utilizes lane optical flow analysis to cancel out the velocity of the patrol vehicle, ensuring tracking accuracy.

Vehicle Detection & Tracking: Couples YOLOv8 bounding boxes with ByteTrack identifiers to monitor vehicles across consecutive frames.

Adaptive Calibration Thresholds: Differentiates parked vs. momentarily stopped vehicles dynamically based on tracking duration and context.

Plate OCR & Temporal Voting: Extracts license plates via OCR, uses regular expressions for regional format matching, and applies temporal voting across multiple frames to maximize alphanumeric accuracy.

Geospatial & GPS Tagging: Integrates NMEA serial streams via an ESP32 and NEO-6M module to anchor violations to exact WGS84 coordinates.

4. Deployment & Configuration Guide

4.1 Processing Server & Dashboard Setup (Laptop)

To initialize the stream processing infrastructure and the administration web dashboard on the host machine, execute the following PowerShell scripts from the project root directory:

# Step 1: Initialize the Stream Processing Server powershell -ExecutionPolicy Bypass -File deploy/laptop/start_stream_server.ps1 -Port 8088 -ModelPath weights/best.pt -Device cpu -showDisplay # Step 2: Initialize the Dashboard Backend (In a separate terminal window) powershell -ExecutionPolicy Bypass -File deploy/laptop/start_backend.ps1 -DbPath output/db/rtipvd_laptop.db -Port 5000

Network Endpoints Available Post-Launch:
 • Main Dashboard UI: http://localhost:5000
 • API Health Gateway: http://localhost:5000/api/health
 • Stream Ingest Portal: http://localhost:8088/ingest/frame

4.2 Edge Sender Node Setup (Raspberry Pi)

To configure the mobile edge device for streaming video frames and synchronized NMEA data from the GPS module, configure the environment and initiate the sender daemon using the bash environment setup below:

cd ~/Desktop/RTIPVD && source .venv/bin/activate # Environment Variables & Sensor Orchestration export RTIPVD_VIDEO_SOURCE="data/videos/video.mp4" export RTIPVD_STREAM_SERVER_URL="http://<Laptop_IP_Address>:8088/ingest/frame" export RTIPVD_STREAM_SEND_FPS=15 export RTIPVD_STREAM_JPEG_QUALITY=70 export RTIPVD_GPS_ENABLED=true export RTIPVD_GPS_SOURCE="serial" export RTIPVD_GPS_SERIAL_PORT="/dev/ttyUSB0" export RTIPVD_GPS_BAUD_RATE=9600 export PYTHONPATH="/home/rtipvd/Desktop/RTIPVD:${PYTHONPATH:-}" # Normalize line endings and execute node pipelines sed -i 's/\r$//' setup.sh send_stream.sh run_pi.sh chmod +x setup.sh send_stream.sh run_pi.sh bash send_stream.sh

5. API Surface & Data Management

The system exposes structured endpoints for continuous monitoring, telemetry updates, and control room visualization hooks.

| HTTP Method & Endpoint | Service Domain | Functional Description |
| --- | --- | --- |
| GET /api/health | Dashboard Backend | Returns heartbeat, database health, and overall process integrity metrics. |
| GET /api/violations?limit=100 | Dashboard Backend | Queries logged offenses. Supports filtering by timestamp, geofence, and zone details. |
| POST /api/violations | Dashboard Backend | Invoked by main pipeline to commit freshly flagged violations (plates, timestamps, tracks). |
| POST /ingest/frame | Stream Ingester | Receives raw frame payloads and synchronized GPS coordinates from edge assets. |

Operational Notice: Custom static parking restrictions and enforcement zones can be dynamically defined using standard GeoJSON spatial structures (e.g., test_zones.geojson). The core configuration script automatically registers these bounding shapes to cross-reference tracked vehicle vectors with restricted geometries.